# NS-MINDMIX-003-ARCHITECTURE

## High-Level Components

Device Adapter: connects to EEG/EMG device and exposes a stream.

Stream Bus: Lab Streaming Layer (LSL) recommended for unified streaming + sync.

Preprocessing: filtering, bandpower features, artifact detection.

Calibration Wizard: short guided routine to establish baseline + triggers.

Intent Engine: classifiers/regressors + smoothing + hysteresis + confidence gating.

Mapping Engine: intent → OSC/MIDI with constraints and presets.

DAW/DJ Integration: Ableton/Max/Bitwig routing, macro racks, clip launcher.

Operator UI: live meters, mapping editor, logs, and a panic button.

Demo Mode: scripted pitch-run steps for repeatable demos.

## Recommended backbone

Use Lab Streaming Layer (LSL) to stream, receive, synchronize, and record neural/physio/behavioral data across tools.