# NS-MINDMIX-005-DECISION-RECORDS

## ADR-001: LSL as default stream backbone

LSL provides a standardized way to stream, synchronize, and record neural/physio/behavioral data across producers/consumers.

## ADR-002: Hybrid input is allowed (and encouraged)

EEG for state + EMG/blinks for reliable triggers. The goal is a strong telekinetic performance feel, not purity.

## ADR-003: Explainability-first

When an action fires, show “why”: thresholds, confidence, smoothing state, and any gating conditions.

## ADR-004: Stage safety over cleverness

Rate-limit automation, require confidence thresholds, and provide a hard panic reset that returns the set to neutral.